# Create a ROS2 package for precision landing

## Go to your ROS 2 workspace

cd ~/ros2_ws/src

If ros2_ws doesn’t exist:

mkdir -p ~/ros2_ws/src

cd ~/ros2_ws

colcon build

source install/setup.bash

cd src

## Create a Python package

ros2 pkg create drone_vision --build-type ament_python --dependencies rclpy sensor_msgs geometry_msgs cv_bridge

This creates:

drone_vision/

├── drone_vision/

│    └── __init__.py

├── package.xml

├── setup.py

└── setup.cfg

## Add your node file

Create the node file:

cd drone_vision/drone_vision

touch bottom_dot_tracker.py

chmod +x bottom_dot_tracker.py

## Paste the Python code

Open the file:

nano bottom_dot_tracker.py

Paste this complete working node:

#!/usr/bin/env python3

import rclpy

from rclpy.node import Node

from sensor_msgs.msg import Image

from geometry_msgs.msg import Vector3

from cv_bridge import CvBridge

import cv2

import numpy as np

class QRDetectorNode(Node):

def __init__(self):

super().__init__('qr_detector_node')

self.subscription = self.create_subscription(

Image,

'/drone/bottom/image_raw',

self.image_callback,

10

)

self.xy_pub = self.create_publisher(

Vector3,

'/landing/xy_error',

10

)

self.bridge = CvBridge()

self.qr_detector = cv2.QRCodeDetector()

self.deadband_px = 20

self.get_logger().info(" QR detector node started")

def image_callback(self, msg):

frame = self.bridge.imgmsg_to_cv2(msg, 'bgr8')

frame = cv2.flip(frame, 0)

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

data, bbox, _ = self.qr_detector.detectAndDecode(gray)

if bbox is None or not data:

return

bbox = bbox[0]

cX = int(np.mean(bbox[:, 0]))

cY = int(np.mean(bbox[:, 1]))

h, w = gray.shape

mid_x = w // 2

mid_y = h // 2

error_x = mid_x - cX

error_y = mid_y - cY

msg = Vector3()

msg.x = float(error_x)

msg.y = float(error_y)

msg.z = 0.0

self.xy_pub.publish(msg)

self.get_logger().info(

f"QR [{data}] | ex={error_x} ey={error_y}"

)

def main():

rclpy.init()

node = QRDetectorNode()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

## Register the node in setup.py

Open setup.py:

nano ../setup.py

Modify only the entry_points section:

entry_points={

'console_scripts': [

'bottom_dot_tracker = drone_vision.bottom_dot_tracker:main',

],

},

This makes the node executable via ros2 run.

## Verify package.xml

Open:

nano ../package.xml

Make sure these exist:

<exec_depend>rclpy</exec_depend>

<exec_depend>sensor_msgs</exec_depend>

<exec_depend>geometry_msgs</exec_depend>

<exec_depend>cv_bridge</exec_depend>

## Build the package

cd ~/ros2_ws

colcon build --packages-select drone_vision

source install/setup.bash

## Run the node

First, make sure Gazebo / drone is running.

Then:

ros2 run drone_vision bottom_dot_tracker

Similarly do is for precision landing code

## RUN SEQUENCE (IMPORTANT)

### Terminal 1

ros2 run drone_vision bottom_dot_tracker

### Terminal 2

ros2 run precision_landing precision_landing

EXPECTED DRONE BEHAVIOR

✔ Aligns over QR
 ✔ Slowly descends
 ✔ Locks XY below 1m
 ✔ Descends vertically
 ✔ Stops at ground